UHF Reader Demo V2.0 Quick Start

One, Connection & Configuration

1, Testing demo supports TCP/IP and RS232 connection. After connect success, reader Firmware version, SN and other parameters info will displayed on System Prompt. If no these info, that means connection failed.

Note:

A: Default IP is 192.168.1.200, Port fixed 100

(should fixed PC’s IP address as Static IP)

B: Default Baud rate is 115200

2, After connect reader success, You can query and do new setting of reader parameters. ( MAC and SN should Select first). After set new parameters, need RsStart to make new parameters available. Then you need connect reader again.

3, MarktraceRFID UHF reader model MR6211E, MR6221E and MR6261E is integrated reader, only Antenna 1 available.

RF power range from 0 to 30. 0dBm is the minimum reading range 0m while 30dBm is maximum reading range.

4,Set Frequency

Some country use specific frequency, for example, Malaysia ISM919-923MHz and Philippines ISM918-920MHz. You can select the frequency on Others.

5, Programmable I/O Port can set Relay ON/OFF

6, Reader support three type of working mode: Command, Trigger and Timing.

Command mode: use command to control reader operation. The testing demo under Command mode.

Trigger mode: Once the trigger cable short circuit, reader will start reading. After the rigger time finished, stop reading.

Timing mode: Once reader got power, it will start reading.

7, MarktraceRFID UHF reader supports EPC Gen2 tag only. Choose the memory section like EPC, TID, Reserved or User if needed. Configure data bytes by Start Address and Length.

8, The start address. For example, if set 0, that means output data from byte 0 of EPC code.

Length means length of output data. Wiegand26 is 3 bytes, RS485 and Wiegand34 are 4 bytes.

9, Filter means filter same tag ID. The Filter time means time of filter data. For example, set filter time 200s, within 200s, reader will output same tag ID once. Filter time range 0 to 255s.

10, Time Interval means Reading card Interval, function is within set time interval, reader will reading cards.The parameters working on Timing mode. For example, set 100ms, reader will start reading every 100ms. Interval range 0 to 255ms.

11, The Interval of Trigger is 0 to 255s

- Data output can in ASCII code and Hex format；

ASCII Code Format

Hex Format

FAQ：

The typical application of UHF reader is parking system and vehicle tracking. So most of customer use RFID reader and controller by Wiegand interface together. As some EPC tags has part of same number on EPC bank. Here is a sample to help you set output different tag ID from Wiegand.

Ag,

Tag ID E2  00  00  19  96  14  01  53  27  80  7F  48

Byte  0   1   2   3  4    5   6   7   8   9   10  11

Total 12 bytes

If set Start Address is 0, wiegand 26 will out put E2 00 00

The First 1, output 00 00 19

Two, Tag Operation

1, Inventory will reading all tag EPC data.

2, Reading can read selected memory section of all tags.

3, Writing can write same data to all tags.

Tips: Reading and Writing supports 96bits EPC tag only.

4, Signal tag operation is for one tag read, write, lock and kill.

Security read first, if you want to write new data into tag, you can use new data cover current data and SecWrite.

Security Lock should write new password to Reserved Access Password. Default password is 00000000

New Access Password is 55556666

Security Lock Access Password to avoid anyone to change password. After that you can lock EPC or User bank.

The third is Security Lock

First choose is Unlock

Second choose is Unlock forever

Third choose is Security lock

Last choose is Lock forever

After Security Lock, should use the correct password to Write tags. If the password was wrong, tag write will be failed.

Reserve byte 0 and byte 1 is kill tag password and byte 2 and byte 3 is security password. If you want to kill tag, need write kill tag password first.